# Making Target Stands

Using the attached drawing as a reference the target stands can be built out of l"x 4" fence slats.

If a lighter stand is wanted ¾"plywood cut into 1" strips can be used. The advantage to this stand is that the weight is approximately 1/3 of the I" x 4" stand.

Board "A" is cut to a 6-foot length

Board "B" is cut to a 5-foot length

Board "C" is cut to a 3-foot length.

Each stand will need 2 of each of the cut board lengths.

## Hardware

The hardware that is needed to assemble the stand can be found at any Home Depot or Hardware Store.

If you are using the plywood to build the stand use 1/8" bolts instead of the ¼" bolts

6-¼"x2½" hex head bolts

4 - ¼" lock nuts

2 - ¼" wing nuts (use the wing nuts on the front of the stand where board C overhangs)

12 - ¼" flat washers

1 strap hinge

To bolt the hinge to the stand use a bolt size that fits though the holes in the hinge. Place the bolts so when the stand is folded the bold beads will not touch.

We are currently looking at using piece& of rubber bungie cord' leather, and rubber hose instead of a hinge.

In using these objects the cost of making the stand is reduced.

Construction: it is assumed that all holes are at the center of the board.

## Board "A"

There are 3 holes in this board these holes are measured from the same end. They are at 2", 10", and 34".

## Board "B"

There are 2 holes in this board these holes are measured from the same end. They are at 5"' 28".

Hinge - The hinge is placed on Board "B". Place the hinge or other  material in the darkened area shown on the drawing.

Board "C" this board will support the target foam and the target nice. There are 2 holes in this board.

Drill one hole 1" from the end.

Drill one hole 5 ½" from the other end.

The above will build ½ of the stand using the binge to join the half together will form the complete stand.

In order to keep the stand together during transportation use a bungee cord to hold stand together. The bungee cord will also assist to hold the target foam and target face to the stand during use.